#include <WiFi.h>

#include <HTTPClient.h>

#include <SPI.h>

#include <MFRC522.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <time.h>

#include <vector>

// RFID pins

#define SS_PIN 5

#define RST_PIN 4

MFRC522 rfid(SS_PIN, RST_PIN);

// OLED setup

#define SCR_W 128

#define SCR_H 64

Adafruit_SSD1306 oled(SCR_W, SCR_H, &Wire, -1);

// my wifi details

const char* ssid     = "YOU USERNAME";

const char* password = "YOUR PASSWORD";

// google sheet web app

String sheetURL = "YOUR WEBAPP URL";

String mySection = "B";

// store records when wifi is off

struct AttendRecord {

String roll;

String cardUID;

String stdName;

String scanDate;

String scanDay;

String scanTime;

};

std::vector<AttendRecord> savedRecords;

bool isWifiOn = false;

unsigned long wifiCheckTimer = 0;

// draw a single wifi arc

void drawArc(int cx, int cy, int r)

{

for(int i = -r; i <= r; i++)

{

int yOff = (i * i) / (r * 2);

oled.drawPixel(cx + i, cy - r + yOff, SSD1306_WHITE);

}

}

// draw wifi icon with signal level 0-3

void drawWiFiIcon(int cx, int cy, int bars)

{

oled.fillCircle(cx, cy, 2, SSD1306_WHITE);

if(bars >= 1) drawArc(cx, cy - 2, 6);

if(bars >= 2) drawArc(cx, cy - 4, 10);

if(bars >= 3) drawArc(cx, cy - 6, 14);

}

void showConnecting(int f)

{

oled.clearDisplay();

drawWiFiIcon(64, 40, f % 4);

oled.setCursor(25, 55);

oled.print("Connecting...");

oled.display();

}

void showConnected()

{

oled.clearDisplay();

drawWiFiIcon(64, 40, 3);

oled.setCursor(40, 55);

oled.print("Connected");

oled.display();

delay(1200);

}

void showNoWifi()

{

oled.clearDisplay();

drawWiFiIcon(64, 40, 3);

// cross over the icon

oled.drawLine(48, 14, 80, 46, SSD1306_WHITE);

oled.drawLine(80, 14, 48, 46, SSD1306_WHITE);

oled.setCursor(10, 55);

oled.print("WiFi Not Connected");

oled.display();

delay(2000);

}

// animated scan screen

void showScanScreen(int f)

{

oled.clearDisplay();

oled.drawRect(54, 20, 20, 14, SSD1306_WHITE);

oled.drawLine(58, 24, 70, 24, SSD1306_WHITE);

int w = f % 3;

oled.drawCircle(64, 27, 8  + w * 3, SSD1306_WHITE);

oled.drawCircle(64, 27, 14 + w * 3, SSD1306_WHITE);

oled.setCursor(28, 54);

oled.print("Scan Your Card");

oled.display();

}

void showUnknownCard()

{

oled.clearDisplay();

oled.drawCircle(64, 22, 16, SSD1306_WHITE);

oled.drawLine(56, 14, 72, 30, SSD1306_WHITE);

oled.drawLine(72, 14, 56, 30, SSD1306_WHITE);

oled.setCursor(26, 50);

oled.println("Unknown Card");

oled.display();

delay(2000);

}

void showWelcome(String stdName)

{

oled.clearDisplay();

oled.setCursor(40, 44);

oled.println("Welcome");

oled.setCursor(20, 56);

oled.println(stdName);

oled.display();

delay(1500);

}

void showStudentInfo(String stdName, String roll, String sec, String t)

{

oled.clearDisplay();

oled.setCursor(0, 10);

oled.print("Name : "); oled.println(stdName);

oled.setCursor(0, 25);

oled.print("Roll : "); oled.println(roll);

oled.setCursor(0, 40);

oled.print("Sec  : "); oled.println(sec);

oled.setCursor(0, 55);

oled.print("Time : "); oled.println(t);

oled.display();

delay(2000);

}

void sendToSheet(AttendRecord &rec)

{

HTTPClient http;

http.begin(sheetURL);

http.addHeader("Content-Type", "application/x-www-form-urlencoded");

String data = "rollno=" + rec.roll +

"&uid="   + rec.cardUID +

"&name="  + rec.stdName +

"&sec="   + mySection +

"&date="  + rec.scanDate +

"&day="   + rec.scanDay +

"&time="  + rec.scanTime;

http.POST(data);

http.end();

}

void setup()

{

Serial.begin(115200);

SPI.begin();

rfid.PCD_Init();

Wire.begin(21, 22);

oled.begin(SSD1306_SWITCHCAPVCC, 0x3C);

oled.setTextSize(1);

oled.setTextColor(SSD1306_WHITE);

// try connecting wifi for 7 seconds

WiFi.begin(ssid, password);

unsigned long t0 = millis();

int f = 0;

while(WiFi.status() != WL_CONNECTED && millis() - t0 < 7000)

{

showConnecting(f++);

delay(400);

}

if(WiFi.status() == WL_CONNECTED)

{

isWifiOn = true;

showConnected();

}

else

{

isWifiOn = false;

showNoWifi();

}

// IST timezone = UTC+5:30 = 19800 seconds offset

configTime(19800, 0, "time.google.com");

}

void loop()

{

static int frameCount = 0;

showScanScreen(frameCount++);

delay(80);

// check if wifi dropped

if(WiFi.status() != WL_CONNECTED && isWifiOn)

{

isWifiOn = false;

showNoWifi();

}

// retry wifi every 10 seconds

if(!isWifiOn && millis() - wifiCheckTimer > 10000)

{

WiFi.begin(ssid, password);

wifiCheckTimer = millis();

}

// wifi came back

if(WiFi.status() == WL_CONNECTED && !isWifiOn)

{

isWifiOn = true;

showConnected();

}

// upload any saved offline records

if(isWifiOn && savedRecords.size() > 0)

{

for(auto &r : savedRecords)

{

sendToSheet(r);

delay(300);

}

savedRecords.clear();

}

// wait for card

if(!rfid.PICC_IsNewCardPresent()) return;

if(!rfid.PICC_ReadCardSerial())   return;

// read card UID

String uid = "";

for(byte i = 0; i < rfid.uid.size; i++)

{

if(rfid.uid.uidByte[i] < 0x10) uid += "0";

uid += String(rfid.uid.uidByte[i], HEX);

}

uid.toUpperCase();

// match uid to student

String stdName = "Unknown";

String roll    = "";

if(uid == "YOUR_RFID_UID")

{

stdName = "WHAT_NAME_YOU_NEED_TO_ASSIGN";

roll    = "ASSIGN_AS_YOU_WANT";

}

else if(uid == "YOUR_RFID_UID")

{

stdName = "WHAT_NAME_YOU_NEED_TO_ASSIGN";

roll    = "ASSIGN_AS_YOU_WANT";

}

if(stdName == "Unknown")

{

showUnknownCard();

rfid.PICC_HaltA();

rfid.PCD_StopCrypto1();

return;

}

// get current time

String dateStr = "OFFLINE";

String dayStr  = "OFFLINE";

String timeStr = "OFFLINE";

if(isWifiOn)

{

struct tm tInfo;

if(getLocalTime(&tInfo))

{

char d[15], da[12], t[10];

strftime(d,  sizeof(d),  "%d-%m-%Y", &tInfo);

strftime(da, sizeof(da), "%A",       &tInfo);

strftime(t,  sizeof(t),  "%H:%M:%S", &tInfo);

dateStr = d;

dayStr  = da;

timeStr = t;

}

}

showWelcome(stdName);

showStudentInfo(stdName, roll, mySection, timeStr);

// build record

AttendRecord rec;

rec.roll     = roll;

rec.cardUID  = uid;

rec.stdName  = stdName;

rec.scanDate = dateStr;

rec.scanDay  = dayStr;

rec.scanTime = timeStr;

if(isWifiOn)

{

HTTPClient http;

http.begin(sheetURL);

http.addHeader("Content-Type", "application/x-www-form-urlencoded");

String data = "rollno=" + rec.roll +

"&uid="   + rec.cardUID +

"&name="  + rec.stdName +

"&sec="   + mySection +

"&date="  + rec.scanDate +

"&day="   + rec.scanDay +

"&time="  + rec.scanTime;

int code = http.POST(data);

if(code <= 0)

savedRecords.push_back(rec); // save if failed

http.end();

}

else

{

savedRecords.push_back(rec); // save for later

}

rfid.PICC_HaltA();

rfid.PCD_StopCrypto1();

delay(3000);

}